ECG Machine (3 Channel)

| S.N. | Purchaser’s Specifications |  |
| --- | --- | --- |
|  | ECG Machine, (3 Channel) |  |
|  | Manufacturer |  |
|  | Brand |  |
|  | Type/Model |  |
|  | Country of Origin |  |
|  | Manufacturer’s Authorization Letter |  |
| 1 | Description Of Function |  |
| 1.1 | ECG Machine is primary equipment to record ECG Signal in various configurations. |  |
| 2 | Operational Requirements |  |
| 2.1 | Portable digital ECG machine must be able to acquire Simultaneous 3 channel ECG recording with 12 lead simultaneous acquisition with auto summary |  |
| 3 | System Configuration |  |
| 3.1 | Portable digital ECG machine with complete accessories |  |
| 4 | Technical Specifications |  |
| 4.1 | Simultaneous acquisition of up to 12 leads |  |
| 4.2 | Should have minimum 3.5 inch TFT Color LCD Display |  |
| 4.3 | Display to Preview signal quality prior to printing thereby saving time and paper |  |
| 4.4 | Should have patient data entry feature |  |
| 4.5 | Should have colour coded keys for ease of operation |  |
| 4.6 | Should have different mode of printing: Automatic, Manual |  |
| 4.7 | Should have different sensitivity levels : 2.5,5,10,20,40 mm/mV Auto |  |
| 4.8 | Recording speeds of 5,10, 25 and 50 mm/sec |  |
| 4.9 | Should have a high frequency Response: 0.05 Hz to 150 Hz |  |
| 4.10 | Should have a sampling frequency of 1000 Hz |  |
| 4.11 | User selectable filter: AC Filter, EMG filter- 25 or 35, Base Line Filter |  |
| 4.12 | At least 80 mm (W) x 20 mm (L) paper width for the thermal printer |  |
| 4.13 | Printer must compatible with Roll ECG paper |  |
| 4.14 | Light weight – Less than 1.6 Kg |  |
| 4.15 | Battery operation – Lithium Ion Battery -minimum 10 HRS continuous work, atleast 3 hours of print and at least 250 ECG out with fully charged battery |  |
| 4.16 | Easy to carry handle |  |
| 4.17 | Automatic measurement and interpretations of ECG data. |  |
| 4.18 | Facility to store at least 1000 ECG data |  |
| 4.19 | Should have pacemaker detection facility |  |
| 4.20 | PC interface facility and optional PC interface software (Optional ECG data transfer feature) |  |
| 4.21 | External storing and retrieving facility through USB storage device |  |
| 5 | Accessories, spares and consumables |  |
| 5.1 | Accessories: |  |
|  | Power Cable – 1 no; |  |
|  | Lead Patient Cable – 1set; |  |
|  | Chest Electrodes – 6 nos; |  |
|  | Clip-on electrodes – 4 nos; |  |
|  | ECG Gel – 2 bottle; |  |
|  | Thermal recording Paper – 5nos |  |
|  | All standard accessories, consumables and parts required to operate the equipment, including all standard tools and cleaning and lubrication materials, to be included in the offer. Bidders must specify the quantity of every item included in their offer (including items not specified above). |  |
| 6 | Operating Environment |  |
| 6.1 | The system offered shall be designed to be stored and to operate normally under the conditions of the purchaser's country. The conditions include Power Supply, Climate, Temperature, Humidity, etc. |  |
| 6.2 | Power supply: 220–240V AC, 50Hz fitted with appropriate plug type. The power cable must be at least 3 metre in length |  |
| 7 | Standards and Safety Requirements |  |
| 7.1 | Must submit ISO13485:2003/AC:2007 for Medical Devices AND |  |
| 7.2 | CE (93/42 EEC Directives) and USFDA approved product certificate |  |
| 8 | User Training |  |
| 8.1 | Must provide user training (including how to use and maintain the equipment). |  |
| 9 | Warranty |  |
| 9.1 | Comprehensive warranty for 1 years after acceptance |  |
| 10 | Maintenance Service During Warranty Period |  |
| 10.1 | During the warranty period supplier must ensure corrective/breakdown maintenance whenever required. |  |
| 11 | Installation and Commissioning |  |
| 11.1 | Supplier must accomplish proper installation and commissioning of the equipment on site. |  |
| 12 | Documentation |  |
| 12.1 | User (Operating) manual in English |  |
| 12.2 | Original Brochure Must be submitted |  |