X-ray Dark Room Accessories

| S.N. | Purchaser’s Specifications |
| --- | --- |
|  | X-ray Dark Room Accessories |
|  | Manufacturer |
|  | Brand |
|  | Type / Model |
|  | Country of Origin |
| 1 | Description of Function |
| 1.1 | X-Ray dark room accessories set for use with X-ray machine. |
| 2 | Operational Requirements |
| 2.1 | The dark room accessories are essential for smooth operation of the x-ray machine. Dark room accessories shall include X-ray cassettes, X-ray hangers, Processing tank, Lead apron, Lead letters, Dark room safe lamp, Patient position pad, X-ray view box, Grids, Gonad shield, and Chest stand. |
| 3 | System Configuration |
| 3.1 | The system consists of: - X-ray cassettes of different sizes - X-ray hangers of different sizes - Processing tank for films - Lead letters - X-ray view box - Lead apron - Patient position pad - Safe light - Grids - Chest stand - Gonad Shield |
| 4 | Technical Specifications |
| 4.1 | X-ray Cassettes: X-ray cassettes, green emitting intensifying screen high speed: - Cassette size 20x24 cm (8 x 10”): 2 Pieces. - Cassette size 24x30 cm. (10 x 12”): 2 Pieces. - Cassette size 30x37 cm (12 x 15”): 4 Pieces. - Cassette size 35x43 cm (14 x 17”): 2 Pieces. |
| 4.2 | X-ray Hangers: X-ray hangers with stainless steel clip: - Hanger size20x24 cm (8 x 10”): 6 Pieces. - Hanger size 24x30 cm (10 x 12”): 6 Pieces. - Hanger size 30x37 cm (12 x 15”): 6 Pieces. - Hanger size 35x43 cm (14 x 17”): 6 Pieces. |
| 4.3 | X-ray Processing Tanks: Stainless steel X-ray processing tank for films up to 35x43cm (14 x17"), approximately 13 litres capacity: 2 Pieces. |
| 4.4 | Lead letters: - Lead letters, A-Z: 1 Set. - Lead figures 0-9 with R & L letter: 10 Set. |
| 4.5 | X-ray View Box with LED illumination: - X-ray view box for single film, size 35x43cm (14x17"): 1 Piece. - X-ray view box for double film: 1 Piece. |
| 4.6 | Lead Apron with Stand: - Lead apron coat type 0.25mm lead with hanger: 1 Piece. - Double sided lead apron 025mm lead: 1 Piece. - Stainless steel lead apron stand for 5 aprons, floor model: 1 Piece. |
| 4.7 | Patient Position Pad: X-ray radiolucent patient position pad: 4 Pieces. |
| 4.8 | Safe light: Dark room safe lamp: 1 Piece. |
| 4.9 | Grids: Stationary grids ratio > 6:1 & gridlines > 80/inch: 2 Pieces. |
| 4.10 | Chest Stand: X-ray chest stand, floor model: 1 Pieces |
| 4.11 | Gonad Shield: Children gonad shield, (male/female): 1 set each. |
| 5 | Accessories, spares and consumables |
| 5.1 | Not applicable. |
| 6 | Operating Environment |
| 6.1 | The product offered shall be designed to store and to operate normally under the conditions of the purchaser's country. The conditions include Climate, Temperature, Humidity, etc. |
| 6.1 | Dark room safe lamp shall operate on 220-240VAC 50Hz power supply, fitted with appropriate plug. The power cable must be minimum 3 metres long. |
| 7 | Standards and Safety Requirements |
| 7.1 | Must submit ISO 9001 or ISO 13485:2003/AC: 2007. |
| 8 | User Training |
| 8.1 | Not applicable |
| 9 | Warranty |
| 9.1 | Warranty for 1 year. |
| 10 | Maintenance Service During Warranty Period |
| 10.1 | Standard warranty conditions are applicable. |
| 11 | Installation and Commissioning |
| 11.1 | Must supply preassembled unit, ready to use. |
| 12 | Documentation |
| 12.1 | User’s (Operating) manual shall supply in English. |