Colorimeter

| S.N. | Purchaser’s Specifications |
| --- | --- |
|  | Colorimeter |
|  | Manufacturer |
|  | Brand |
|  | Type / Model |
|  | Country of Origin |
| 1 | Description of Function |
| 1.1 | General purpose colorimeter use in clinical laboratory. |
| 2 | Operational Requirements |
| 2.1 | Microprocessor controlled system. |
| 3 | System Configuration |
| 3.1 | Colorimeter with complete accessories. |
| 4 | Technical Specifications |
| 4.1 | Must have 8 no of filters wave length from 400 nm to 700 nm. |
| 4.2 | Must have a 3 digit LED display calibrated directly in optical density. |
| 4.3 | Detector must be encased spill proof photocell. |
| 4.4 | Must have facilities for concentration, calculation, percentage transmission and optical density. |
| 4.5 | Lamp source: Broad spectrum LED, covering full visible range |
| 5 | Accessories, spares and consumables |
| 5.1 | Accessories: - Turret-mounted filters - Cuvettes: 10 nos. - Test tube stand: 02 nos. |
| 5.2 | All standard accessories, consumables and parts required to operate the equipment, including all standard tools and cleaning and lubrication materials, to be included in the offer. Bidders must specify the quantity of every item included in their offer (including items not specified above). |
| 6 | Operating Environment |
| 6.1 | The system offered shall be designed to operate normally under the conditions of the purchaser's country. The conditions include Power Supply, Climate, Temperature, Humidity, etc. |
| 6.2 | Power supply: 220 – 240 VAC, 50Hz fitted with appropriate plug. The power cable must be at least 3 metre in length. |
| 7 | Standards and Safety Requirements |
| 7.1 | Must submit ISO13485:2003/AC:2007 for Medical Devices AND |
| 7.2 | CE (93/42 EEC Directives) or USFDA approved product certificate. |
| 7.3 | Shall meet IEC 61010-1 safety requirements for electrical equipment for measurement, control, and laboratory use. |
| 8 | User Training |
| 8.1 | Must provide user training (including how to use and maintain the equipment). |
| 9 | Warranty |
| 9.1 | Comprehensive warranty for 1 year after acceptance. |
| 10 | Maintenance Service During Warranty Period |
| 10.1 | During the warranty period supplier must ensure corrective/breakdown maintenance whenever required. |
| 11 | Installation and Commissioning |
| 11.1 | Supplier must accomplish proper installation and commissioning of the equipment on site. |
| 12 | Documentation |
| 12.1 | User (Operating) manual in English. |
| 12.2 | Service (Technical / Maintenance) manual in English. |
| 12.3 | Certificate of calibration and inspection. |
| 12.4 | List of important spare parts and accessories with their part numbers and costing. |