Digital pH Meter

| S.N. | Purchaser’s Specifications |
| --- | --- |
|  | Digital pH Meter |
|  | Manufacturer |
|  | Brand |
|  | Type / Model |
|  | Country of Origin |
| 1 | Description of Function |
| 1.1 | A pH meter is an electronic instrument used to measure the pH (acidity or basicity) of a blood sample in blood bank. |
| 2 | Operational Requirements |
| 2.1 | Portable digital pH meter is required. |
| 3 | System Configuration |
| 3.1 | Digital pH Meter with complete accessories. |
| 4 | Technical Specifications |
| 4.1 | pH range must be 0-14. |
| 4.2 | Mile volt range must be 0-1999+/- mv. |
| 4.3 | Resolution 0.01 pH. |
| 4.4 | Repeatability +/- 0.01pH +/- 1 digit. |
| 4.5 | Standardization range +/- 2 pH. |
| 4.6 | Temperature compensation 0-100 0C. |
| 4.7 | Display 4 digit LED with automatic polarity. |
| 4.8 | Polarizing current 10 micro amperes. |
| 4.9 | Dimensions: Approx. 240x230x115 mm. |
| 4.10 | Weight approx. 2 kg. |
| 5 | Accessories, spares and consumables |
| 5.1 | Accessories: - Part of glass and reference electrodes - Electrode holder |
| 5.2 | All standard accessories, consumables and parts required to operate the equipment, including all standard tools and cleaning and lubrication materials, to be included in the offer. Bidders must specify the quantity of every item included in their offer (including items not specified above). |
| 6 | Operating Environment |
| 6.1 | The product offered shall be designed to be stored and to operate normally under the conditions of the purchaser's country. The conditions include Power Supply, Climate, Temperature, Humidity, etc. |
| 6.2 | Power supply: 220 – 240 VAC, 50Hz fitted with appropriate plug. The power cable must be at least 3 metre in length. |
| 7 | Standards and Safety Requirements |
| 7.1 | Must submit ISO13485:2003/AC:2007 for Medical Devices AND |
| 7.2 | CE (93/42 EEC Directives) or USFDA approved product certificate. |
| 7.3 | Shall meet IEC 61010-1 safety requirements for electrical equipment for measurement, control, and laboratory use. |
| 8 | User Training |
| 8.1 | Must provide user training (including how to use and maintain the equipment). |
| 9 | Warranty |
| 9.1 | Comprehensive warranty for 2 years after acceptance. |
| 10 | Maintenance Service During Warranty Period |
| 10.1 | During the warranty period supplier must ensure corrective/breakdown maintenance whenever required. |
| 11 | Installation and Commissioning |
| 11.1 | The bidder must arrange for the equipment to be installed and commissioned by certified or qualified personnel; any prerequisites for installation to be communicated to the purchaser in advance, in detail. |
| 12 | Documentation |
| 12.1 | User (Operating) manual in English. |
| 12.2 | Service (Technical / Maintenance) manual in English. |
| 12.3 | List of important spare parts and accessories with their part numbers and costing. |
| 12.4 | Certificate of calibration and inspection from factory. |