Wheel Chair (Hi-Tech)

| S.N. | Purchaser’s Specifications |  |
| --- | --- | --- |
|  | Wheel Chair (Hi-Tech) |  |
|  | Manufacturer |  |
|  | Brand |  |
|  | Type/Model |  |
|  | Country of Origin |  |
| 1 | Description of Function |  |
| 1.1 | Patients with disability must be well positioned in their wheel chair to be comfortable and for their seating to support them correctly. |  |
| 2 | Operational Requirements |  |
| 2.1 | To be used in physiotherapy department. Wheel chair posture aims to: - Spread pressure evenly to ensure comfort and prevent development of pressure sores or pain. - Minimize fatigue. - Minimize the development of muscle tightness and other deformities. - Limit abnormal tone patterns, such as, pushing back into extension. |  |
| 3 | System Configuration |  |
| 3.1 | Hi-Tech wheel chair with tilt in space option. |  |
| 4 | Technical Specifications |  |
| 4.1 | Tire sizes: Rear approx. 600mm (24") solid Mag tyres or Bicycle type spoked wheels, and Front approx. 200mm (8") Mag swivel casters. |  |
| 4.2 | The seats with a width of approximately 40cm and depth of 45cm. |  |
| 4.3 | The folded width of the wheel chair must be 33cm. |  |
| 4.4 | The armrest height must not exceed 26cm for patient comfort. |  |
| 4.5 | The seat height must be of maximum 50cm height. |  |
| 4.6 | The overall height of the wheelchair must be maximum 140cm for easy portability. |  |
| 4.7 | The chair weight must be approximately 17.5 kg for easy carrying and portability. |  |
| 4.8 | The maximum weight that the wheelchair can carry must be around 120kg. |  |
| 4.9 | The tilting angle of the wheelchair must be from 0-35 degrees. |  |
| 4.10 | The wheelchair must have removable footrests. |  |
| 4.11 | The headrest must be foldable and height adjustable. |  |
| 5 | Accessories, spares and consumables |  |
| 5.2 | All standard accessories, consumables and parts required to operate the equipment, including all standard tools and cleaning and lubrication materials, to be included in the offer. Bidders must specify the quantity of every item included in their offer (including items not specified above). |  |
| 6 | Operating Environment |  |
| 6.1 | The system offered shall be designed to store and to operate normally under the conditions of the purchaser's country. The conditions include Climate, Temperature, Humidity, etc. |  |
| 7 | Standards and Safety Requirements |  |
| 7.1 | Must submit ISO 9001 or ISO13485:2003/AC:2007 AND |  |
| 7.2 | CE or USFDA approved product certificate. |  |
| 8 | User Training |  |
| 8.1 | Must provide user training (including how to use and maintain the equipment). |  |
| 9 | Warranty |  |
| 9.1 | Comprehensive warranty for 1 year. |  |
| 10 | Maintenance Service During Warranty Period |  |
| 10.1 | Standard warranty conditions are applicable. |  |
| 11 | Installation and Commissioning |  |
| 11.1 | Must supply preassembled unit ready to use. |  |
| 12 | Documentation |  |
| 12.1 | User (Operating) manual in English |  |
| 12.2 | Service (Technical / Maintenance) manual in English |  |
| 12.3 | List of important spare parts and accessories with their part numbers and costing. |  |